Oberkochen, January 2026

Edge Quality without compromise

The Diamaster PRO EdgeExpert ensures flawless furniture fronts – even on sensitive surfaces

Modern furniture fronts combine sophisticated design with high durability. High-gloss surfaces, glass laminates, and decorative coated panels have become indispensable in industrial furniture production. At the same time, these materials place the highest demands on machining: even the smallest chipping, damaged edges, or frayed protective films impair the visual appearance and often result in rework and increased scrap rates. With the Diamaster PRO EdgeExpert, Leitz offers a tooling solution specifically developed for these sensitive applications.

The innovative concept of the Diamaster PRO EdgeExpert combines the strengths of a sizing cutter and a jointing cutter in a single tool. This results in exceptionally high edge quality already during the sizing process – a decisive advantage, as perfect results are achieved in the very first machining step.

Perfect cut edges on all common machine types

The Diamaster PRO EdgeExpert produces chip-free cut edges on console machines, nesting machines, and panel dividing saws – even with demanding surfaces and high-quality top layers. Especially in serial production, this significantly increases process reliability. The protective film is cut cleanly without fraying. Downstream operations can be carried out without additional corrections.

A key quality factor is the precisely coordinated tool geometry. The axial angles are specifically adapted to the different material zones: a higher axial angle in the area of the top layers improves cut quality and minimizes tear-out, while a lower axial angle in the core layer optimizes chip evacuation. At the same time, only minimal pressure is exerted on the core layer, preventing compression and preserving the structural integrity of the material. Large chip gullets ensure reliable chip removal, even at high feed speeds.

Higher feed rates, less rework

The Diamaster PRO EdgeExpert enables feed speeds of up to 25 m/min. This noticeably reduces cycle times and increases efficiency in the production process. Less rework and lower scrap rates further enhance cost-effectiveness. The diamond-tipped tool also sets benchmarks in terms of service life. It can be resharpened up to three times and maintains its performance over extended operating periods. This makes the Diamaster PRO EdgeExpert a sustainable and economically sound investment for companies with the highest quality standards.

The standard range includes two versions with a diameter of 12 mm and cutting lengths of 24 mm or 30 mm. The tool is ideally designed for panel thicknesses from 16 to 28 mm and offers maximum flexibility when machining a wide variety of furniture materials with high-quality surfaces.

The Diamaster PRO EdgeExpert stands for perfect edges, maximum precision, and productivity that delivers measurable benefits directly in production.

The company

Founded in 1876 in Oberkochen, southern Germany, the Leitz Group celebrates its 150th anniversary this year. Today, as a family-owned company in its fifth generation, Leitz is the world’s leading manufacturer of tools for the professional cutting and machining of solid wood, wood-based materials, plastics, composite materials, and non-ferrous metals. The product spectrum covers the full range of machine-driven precision tools and tooling systems. As a manufacturing service provider, Leitz supports its global customers with a comprehensive range of services and consulting, ensuring the optimal application of cutting tools. Leitz products are regularly used in more than 150 countries. Together with its two legally independent sister companies Boehlerit and Bilz, Leitz forms the globally active Brucklacher Group. With a total of 17 production sites, its own sales and service companies in 38 countries across 137 locations and an exclusive partner network, the Brucklacher Group is a global player represented on all continents. The Brucklacher Group employs over 4,000 people worldwide and generates an annual turnover of around 450 million euros.

For more press information and high resolution images, contact:

Lisa Wykydal

Marketing

Phone:	+49 7364 950 - 435

Fax:	+49 7364 950 - 662

E-Mail:	lwykydal@leitz.org

ill. 1: The Diamaster PRO EdgeExpert combines a sizing cutter and a jointing cutter in a single tool, delivering exceptionally high edge quality already during the sizing process. For perfect surfaces without tear-out – even on the most demanding materials. (Photo: Leitz)

ill. 2: Optimized cutting geometry and generously dimensioned chip gullets enable high feed rates while maintaining excellent surface quality. (Photo: Leitz)